GAZIFÈRE INC.

DEMANDES DE RENSEIGNEMENT NO 1 DE LA RÉGIE

CAUSE TARIFAIRE 2001


ENGAGEMENT-19 :
Déposer une version complète de GI-15, document 1.5, comprenant les chiffres pour SCGM.

21. Référence :
GI-15, document 1, 17ème à 23ème pages

Préambule :

Pour les branchements, les conduites principales, les postes de mesurage et les compteurs :

· les durées de vie moyennes proposées sont respectivement de 50, 70, 26 et 11 années

· les taux de dépréciation proposés sont respectivement de 5,55 %, 2,78 %, 4,10 % et 2,92 %

· les « salvage ratios » proposés sont respectivement de -150 %, - 80 %, - 5 % et + 65 % 

Demande :

Veuillez présenter un tableau comparatif des durées de vie moyennes, des taux de dépréciation et des « salvage ratios », pour ces quatre types d’actifs, qui sont utilisés par les principaux distributeurs gaziers canadiens.

Réponse :

Enbridge Consumers Gas

Avg Life(Yrs.)
Dpn Rate(%0)
Salvage(%)

1994 Study





Services




50


4.3


(100)

Mains


Coated Steel




70


3.0


(90)

Cast Iron




50


13.3


(90)

Plastic





70


2.8


(90)

Bare Steel(Other)



35


2.7


(90)

Reg Equip.




23


4.4


(5)

Meters





34


2.9


0

Union Gas



Avg Life(Yrs.)
Dpn Rate(%0)
Salvage(%)
1998 Study

Services

Steel





45


3.93


(60)

Plastic





55


3.21


(75)

Mains


Steel





55


2.35


(25)

Plastic





60


2.36


(40)

Reg. Equip.




25


4.98


(20)

Meters





20


5.56


(5)

Note: The above is applicable to Union’s southern region where over 60% of the assets

Are located.

SaskEnergy Inc. 
Avg Life(Yrs.)
Dpn Rate(%0)
Salvage(%)
Services



Steel



30


3.3


N/A

Plastic



40


2.5


N/A

Mains



40


2.5


N/A

Reg. Equip.


33


3.0


N/A

Meters



29


3.5


N/A

Note: Salvage rates are not identified seperately.

BC Gas 

Avg Life(Yrs.)
Dpn Rate(%0)
Salvage(%)
Services


50


2.0


(50)

Mains



50


2.0


(20)

Reg. Equip.


33


3.0


0

Meters



33


3.0


0

Note: New rates are being proposed (Services 3%, Mains 2.3%, Reg. Equip. 4.7% and Meters 4.8%)


Atco North

(Northwestern Utilities)

(1993 Study)




Avg Life(Yrs.)
Dpn Rate(%0)
Salvage(%)

Services


50


4.64


(100)

Mains



65


2.31


(30)

Reg. Equip.


35


4.50


(30)

Meters



30


3.53


(1)

Atco South

(Canadian Western Natural Gas)

(1998 Study)




Avg Life(Yrs.)
Dpn Rate(%0)
Salvage(%)

Services


50


4.58


(100)

Mains



60


2.94


(60)

Reg. Equip.


38


5.21


(75)

Meters



30


3.47


1


Gaz Metropolitain

(1995 Study)





Avg Life(Yrs.)
Dpn Rate(%0)
Salvage(%)

Services

Steel



34


4.00


(24)

Plastic Insertions

37


3.58


(30)

Copper Insertion

14


20.69


(48)

Plastic Buried


35


3.36


(14)

Aluminium


35


3.24


(18)

Mains

Steel



43


2.77


(9)

Plastic Insertions

50


2.20


(13)

Plastic Buried


50


2.27


(10)

Aluminium


50


2.42


(19)

Reg. Equip.

Stations


32


3.47


(12)

Manholes


22


6.00


(5)

Meters



24


4.50


n/a

Original : 2000-08-18

GI-15

Révisé :  2000-11-03

Document 1.5



Page 1 de 1


Requête 3446-2000


